simion.workbench_program()

adjustable switch_time = 1.7    -- switch time (microseconds)

adjustable buncher_voltage = 900    -- buncher deceleration voltage

-- control buncher voltage

function segment.fast_adjust()

-- set electrode #1 voltage

-- switch off voltage at switch time

adj_elect01 = (ion_time_of_flight < switch_time) and buncher_voltage or 0

end

-- adjust time step

function segment.tstep_adjust()

-- make sure the time step ends right on the switch time

if ion_time_of_flight < switch_time then

ion_time_step = math.min(ion_time_step, switch_time - ion_time_of_flight)

end

end

-- handle display updates

local update_flag = 1   -- flag for updating PE surface display

function segment.other_actions()

if math.abs(ion_time_of_flight - switch_time) < 1e-6 then  -- transition point

ion_color = 3      -- set ion color to blue

mark()             -- mark ion location

update_flag = 1    -- trigger PE surface update at next time step

elseif update_flag == 1 then  -- time step immediately after transition

update_flag = 0

sim_update_pe_surface = 1   -- mark PE display for update

end

end

-- calculate and display results at the end of the simulation

local first_ion = 0    -- flag for first ion

local max_time = 0     -- holds max tof

local min_time = 0     -- holds min tof

function segment.terminate()

if first_ion == 0 then

first_ion = 1

max_time = ion_time_of_flight

min_time = ion_time_of_flight

else

max_time = math.max(ion_time_of_flight, max_time)

min_time = math.min(ion_time_of_flight, min_time)

print("Avg TOF = " .. (max_time + min_time) / 2 ..

"  Delta TOF = " .. (max_time - min_time) ..

" usec")

end

end