--[[

tune.lua

SIMION user program for tune.pa0.

D.Manura 2011-11,2006-08

(c) 2006-2011 Scientific Instrument Services, Inc. (Licensed under SIMION 8.1)

--]]

simion.workbench_program()

simion.import 'contourlib.lua'

-- voltage for central electrode

-- Note: the default Value here was found using the _tune example.

adjustable test_voltage = 0

-- number of grid points to step electrode x location per run.

adjustable step_x = 5

adjustable V1_begin = -2000 -- Source

adjustable V2_begin = -2000   -- elettrodo

adjustable V1_step = 50   -- V1 voltage step size

adjustable V2_step = 50   -- V2 voltage step size

adjustable V1_count = 41  -- V1 voltage number of steps

adjustable V2_count = 41  -- V2 voltage number of steps

adjustable excel_enable = 1  -- Use Excel? (1=yes, 0=no)

local V1_voltage  -- current Value of voltage V1

local V2_voltage  -- current Value of voltage V2

local nhits      -- number of hits detected in current run

local esterni      -- number of hits detected in current run

local angoloInternoMax=0

local angoloI=0

local angoloEsternoMax=0

local angoloEsternoMax1=0

local angoloE=0

local aperturaMax=0

local apertura=0

-- Excel objects.

local excel

local function excel_begin()

excel = simion.import 'C:/Program Files/SIMION-8.1/examples/excel/excellib.lua' . get_excel()

excel.Visible = true

excel_wb = excel.Workbooks:Add()

excel_ws = excel_wb.Worksheets(1)

excel_ws.Cells(1, 1).Value2 = "Number of particles transmitted for each combination of voltages V2,V1."

excel_ws.Cells(2, 1).Value2 = "V2 - V1"

end

local b = 0.465  -- gauss

function segment.mfield_adjust()

ion_bfieldx_gu = 0

ion_bfieldy_gu = 0

ion_bfieldz_gu = b

end

-- Utility for HTML file writing.

local Html = function(filename)

local fh = assert(io.open(filename, "w"))

fh:write('<html><body>\n')

local self = {}

function self:add_image(imgfilename)

fh:write(('<img src="%s" />\n'):format(imgfilename))

end

function self:add_line(text)

fh:write(('<div>%s</div>\n'):format(text))

end

function self:close()

fh:write('</body></html>\n')

fh:close()

end

return self

end

local result_y

-- Performs a simulation on a single parameterization

-- of the geometry.

-- dx is the axial x-offset in grid units.

-- returns calculated y-offset of outermost splat.

local function test(dx)

-- convert GEM file to PA# file.

_G.dx = dx  -- pass V1riable to GEM file.

local pa = simion.open_gem('tune.gem'):to_pa()

-- TODO: this function not yet documented

pa:save('tune.pa#')

pa:close()

-- reload in workbench

simion.wb.instances[1].pa:load('tune.pa#')

-- refine PA# file.

simion.wb.instances[1].pa:refine()

angoloInternoMax=0

angoloI=0

angoloEsternoMax=0

angoloEsternoMax1=0

angoloE=0

-- Fly ions and collect results.

run()

--  print("RESULT: dx=" .. dx .. ", ion_py_gu=" .. 0)

return 0

end

-- called on Fly'm and expected to initiate runs by calling `run()`.

function segment.flym()

-- Initialize Excel speadsheet (if enabled).

if excel_enable ~= 0 then

excel_begin()

end

-- Simulate all parameterizations.

-- Results are summarized in results.csv.

-- enable 'output' directory exists

os.execute("mkdir output")

-- cleanup any old files.

os.remove("output/results.csv")

-- Open output files.

local results_file = assert(io.open("output/results.csv", "w")) -- write mode

results_file:write("dx, dy\n")

local html = Html("output/results.html")

-- run each test, collecting results to results.csv

-- Run simulation.

for V2_pos = 1,V2_count do

for V1_pos = 1,V1_count do

-- Prepare for this run.

V1_voltage = V1_begin + (V1_pos - 1) * V1_step

V2_voltage = V2_begin + (V2_pos - 1) * V2_step

angoloInternoMax=0

angoloI=0

angoloEsternoMax=0

angoloEsternoMax1=0

angoloE=0

aperturaMax=0

apertura=0

-- Perform trajectory calculation run.

run()

-- Record results.

--      print('result: V1='..V1_voltage..',V2='..V2_voltage..',nhits='..nhits..',AngoloI='..angoloInternoMax..'°,esterni='..esterni..',AngoloE='..angoloEsternoMax..'°')

if excel_enable ~= 0 then

excel_ws.Cells(2, V1_pos+1).Value2 = V1_voltage

excel_ws.Cells(V2_pos+2, 1).Value2 = V2_voltage

excel_ws.Cells(V2_pos+2, V1_pos+1).Value2 = nhits

excel_ws.Cells(2+V2_count*2, V1_pos+1).Value2 = V1_voltage

excel_ws.Cells(V2_pos+2+V2_count*2, 1).Value2 = V2_voltage

excel_ws.Cells(V2_pos+2+V2_count*2, V1_pos+1).Value2 = esterni

excel_ws.Cells(2+V2_count*4, V1_pos+1).Value2 = V1_voltage

excel_ws.Cells(V2_pos+2+V2_count*4, 1).Value2 = V2_voltage

excel_ws.Cells(V2_pos+2+V2_count*4, V1_pos+1).Value2 = angoloInternoMax

excel_ws.Cells(2+V2_count*6, V1_pos+1).Value2 = V1_voltage

excel_ws.Cells(V2_pos+2+V2_count*6, 1).Value2 = V2_voltage

excel_ws.Cells(V2_pos+2+V2_count*6, V1_pos+1).Value2 = angoloEsternoMax

excel_ws.Cells(2+V2_count*8, V1_pos+1).Value2 = V1_voltage

excel_ws.Cells(V2_pos+2+V2_count*8, 1).Value2 = V2_voltage

excel_ws.Cells(V2_pos+2+V2_count*8, V1_pos+1).Value2 = aperturaMax

end

-- Print image of screen to file.

simion.printer.type = 'png'  -- 'bmp', 'png', 'jpg'

simion.printer.filename = 'output/result_' .. V1_voltage .. ", " .. V2_voltage .. '.png'

simion.printer.scale = 1

simion.print_screen()

-- caution: print_screen redraws the screen.  If ion trajectories

-- are unsaved (i.e. e.g. sim_rerun_flym == 1), they will be lost.

-- Write data to file.

results_file:write(V1_voltage .. ", " .. V2_voltage .. "\n")

results_file:flush()  -- immediately output to disk

if html then

html:add_image('result_' .. V1_voltage .. ", " .. V2_voltage .. '.png')

html:add_line('result: V1='..V1_voltage..',V2='..V2_voltage..',nhits='..nhits..',AngoloI='..angoloInternoMax..'°,esterni='..esterni..',AngoloE='..angoloEsternoMax..'°'..',aperturaMax='..aperturaMax..'°')

end

end

end

-- Close output files.

results_file:close()

if html then html:close() end

-- Show results.

os.execute([[start notepad output\results.csv]])

if html then

--   print "See also output/results.html."

os.execute([[start output\results.html]]) -- or "firefox"

-- note: need for two "starts" may be a bug in SIMION (TO REVIEW)

end

end

-- called exactly once on run initialization.

function segment.initialize_run()

-- Ensure keeping ion trajectories (required for `simion.print_screen`).

sim_rerun_flym = 0

sim_trajectory_image_control = 0 -- keep trajectories (when rerun 0)

first = true

nhits = 0  -- reset for next run

esterni=0

angoloInternoMax=0

angoloI=0

angoloEsternoMax=0

angoloEsternoMax1=0

angoloE=0

apertura=0

aperturaMax=0

end

-- called on PA initialization to set voltages.

function segment.init_p_values()

-- sets central electrode voltage

adj_elect01 = 0

adj_elect03 = V1_voltage

adj_elect02 = 0

;    adj_elect04 = 0

adj_elect05 = V1_voltage

adj_elect06 = -1000

adj_elect07 = V1_voltage

adj_elect08 = V2_voltage

end

-- called on each particle splat in PA instance

function segment.terminate()

-- tune only on ion #6

if ion_number == 6 then

-- store results

result_y = ion_py_gu

end

-- Acceptance criteria

if ion_px_gu>149.99 and ion_px_gu <150.01 then

nhits = nhits + 1

AngoloI=57.2958*atan((sqrt (ion_py_gu*ion_py_gu+ion_pz_gu*ion_pz_gu))/(ion_px_gu-50))

if aperturaMax<abs(apertura) then

aperturaMax=abs(apertura)

end

end

if ion_px_gu>141 and ion_px_gu <314 and ion_py_gu>-81 and ion_py_gu<-79 then

nhits = nhits + 1

AngoloI=57.2958*atan((sqrt (ion_py_gu*ion_py_gu+ion_pz_gu*ion_pz_gu))/(ion_px_gu-50))

if aperturaMax<abs(apertura) then

aperturaMax=abs(apertura)

end

end

if ion_px_gu>303 and ion_px_gu <305 and ion_py_gu>-81 and ion_py_gu<79 then

nhits = nhits + 1

AngoloI=57.2958*atan((sqrt (ion_py_gu*ion_py_gu+ion_pz_gu*ion_pz_gu))/(ion_px_gu-50))

if aperturaMax<abs(apertura) then

aperturaMax=abs(apertura)

end

end

if ion_px_gu>130 and ion_px_gu <132 and ion_py_gu>19 and ion_py_gu<91 then

esterni = esterni + 1

AngoloE=57.2958*atan((sqrt (ion_py_gu*ion_py_gu+ion_pz_gu*ion_pz_gu))/(ion_px_gu-50))

end

if ion_px_gu>130 and ion_px_gu <132 and ion_py_gu>-91 and ion_py_gu<-19 then

esterni = esterni + 1

AngoloE1=57.2958*atan((sqrt (ion_py_gu*ion_py_gu+ion_pz_gu*ion_pz_gu))/(ion_px_gu-50))

end

if AngoloE == nil then

AngoloE=0

end

if AngoloE1 == nil then

AngoloE1=0

end

if AngoloI == nil then

AngoloI=0

end

if angoloInternoMax<AngoloI then

angoloInternoMax=AngoloI

end

if angoloEsternoMax<AngoloE then

angoloEsternoMax=AngoloE

end

if angoloEsternoMax1<AngoloE1 then

angoloEsternoMax1=AngoloE1

end

end

-- called exactly once on run termination.

function segment.terminate_run()

-- simion.sleep(1) -- optional pause for easier visualization.

sim_rerun_flym = 1 -- clears trajectories on rerun

end

function segment.other_actions()

-- Update the PE surface display on first time-step of run.

if first then first = false; sim_update_pe_surface = 1 end

end

function segment.initialize()

local volts = simion.wb:epotential(ion_px_mm, ion_py_mm, ion_pz_mm)

-- note: ion_volts not accessible from initialize segment; why?

--local eV = V2_voltage

--assert(eV >= 0)

--local new_speed = ke_to_speed(eV, ion_mass)

--local old_speed = rect3d_to_polar3d(ion_vx_mm, ion_vy_mm, ion_vz_mm)

--assert(old_speed ~= 0)

--apertura=0

--ion_vx_mm = ion_vx_mm * new_speed / old_speed

--ion_vy_mm = ion_vy_mm * new_speed / old_speed

--ion_vz_mm = ion_vz_mm * new_speed / old_speed

end

-- optionally draw solenoid wires too